NETWORK_DESIGN_MOHAMMEDZAIN

Design and Implementation of a Hospital System Network Design.

Network Design:

Network Architecture Overview:

I designed a hierarchical network model featuring built-in redundancy to ensure High Availability (fault tolerance) and optimized Load Balancing for peak performance.

Core Infrastructure:

- Site Design: Each site utilizes a three-tier architecture consisting of one Core Router, two Multilayer Switches (Distribution layer), and multiple Access Switches dedicated to specific departments.

- Connectivity: The HQ and Branch networks are linked via a Serial Connection for site-to-site communication.

- Wireless: Integrated wireless networking is available for users across all departments.

VLAN Segmentation & Departments:

The network is logically segmented into 13 VLANs to enhance security and manage traffic

| Location | Departments | VLAN IDs |
| --- | --- | --- |
| HQ Network | MLOCS, MER, MRM, IT, CS, HQ-GWA | 10, 20, 30, 40, 50, 60 |
| Branch Network | NS0, HL, HR, MK, FIN, BR-GWA | 80, 90, 100, 110, 120, 130 |
| Server-side-site (SSS) | DHCP, DNS, Email, and Web Servers | 70 |

IP Addressing:

HQ Hospital IP Addressing

Base Network: 192.168.100.0

| Department | Network Address | Subnet Mask | Host Address Range | Broadcast Address |
| --- | --- | --- | --- | --- |
| MLOCS | 192.168.100.0 | 255.255.255.192/26 | 192.168.100.1 to 192.168.100.62 | 192.168.100.63 |
| MER | 192.168.100.64 | 255.255.255.192/26 | 192.168.100.65 to 192.168.100.126 | 192.168.100.127 |
| MRM | 192.168.100.128 | 255.255.255.192/26 | 192.168.100.129 to 192.168.100.190 | 192.168.100.191 |
| IT | 192.168.100.192 | 255.255.255.192/26 | 192.168.100.193 to 192.168.100.254 | 192.168.100.255 |
| CS | 192.168.101.0 | 255.255.255.192/26 | 192.168.101.1 to 192.168.101.62 | 192.168.101.63 |
| GWA | 192.168.101.64 | 255.255.255.192/26 | 192.168.101.64 to 192.168.101.126 | 192.168.101.127 |

Branch Hospital IP Addressing:

| Department | Network Address | Subnet Mask | Host Address Range | Broadcast Address |
| --- | --- | --- | --- | --- |
| NSO | 192.168.101.128 | 255.255.255.224/27 | 192.168.101.129 to 192.168.101.158 | 192.168.101.159 |
| HL | 192.168.101.160 | 255.255.255.224/27 | 192.168.101.161 to 192.168.101.190 | 192.168.101.191 |
| HR | 192.168.101.192 | 255.255.255.224/27 | 192.168.101.193 to 192.168.101.222 | 192.168.101.223 |
| MK | 192.168.101.224 | 255.255.255.224/27 | 192.168.101.225 to 192.168.101.254 | 192.168.101.255 |
| FIN | 192.168.102.0 | 255.255.255.224/27 | 192.168.102.1 to 192.168.102.30 | 192.168.102.31 |
| GWA | 192.168.102.32 | 255.255.255.224/27 | 192.168.102.33 to 192.168.102.62 | 192.168.102.63 |

Server-Side Site (SSS):

| Department | Network Address | Subnet Mask | Host Address Range | Broadcast Address |
| --- | --- | --- | --- | --- |
| SSS | 192.168.102.64 | 255.255.255.240/28 | 192.168.102.65 – 192.168.102.78 | 192.168.102.79 |

Link Infrastructure & IP Allocation

Router to Layer-3 Switch Interconnects

- These point-to-point links use a /30 mask, providing two usable IP addresses per connection—ideal for connecting routers to multilayer switches.

| Connection ID | Device Link | Network Address |
| --- | --- | --- |
| 1 | HQR1 to HQMLSW1 | 192.168.102.80/30 |
| 2 | HQR1 to HQMLSW2 | 192.168.102.84/30 |
| 3 | BRR1 to BRMLSW1 | 192.168.102.88/30 |
| 4 | BRR1 to BRMLSW2* | 192.168.102.92/30 |
| 5 | HQR1 to BRR1 | 192.168.102.96/30 |

Between the Routers and ISPs:

- Public IP addresses: 195.136.17.0/30, 195.136.17.4/30, 195.136.17.8/30, and 195.136.17.12/30.

CONFIGURATION:

- Department MLOCS:

HOSTNAME – MLOS-SW, MLCOS PRINTER

PASSWORD – MLOCSS

- Department MER:

HOSTNAME – MER-SW, MER PRINTER

PASSWORD – MERR

- Department MRM:

HOSTNAME –MRM-SW, MRM PRINTER

PASSWORD – MRMM

- Department IT:

HOSTNAME – IT-SW, IT PRINTER

PASSWORD – ITT

- Department CS:

HOSTNAME – CS-SW, CS PRINTER

PASSWORD – CSS

- Department HQ-GWA:

HOSTNAME – HQ-GWA-SW, HQ-GWA PRINTER

PASSWORD – HQ-GWAA

- Department SSS:

HOSTNAME – SSS-SW, SSS PRINTER

PASSWORD – SSS

- Department NSO:

HOSTNAME – NSO-SW, NSO PRINTER

PASSWORD – NSOO

- Department HL:

HOSTNAME – HL-SW, HL PRINTER

PASSWORD – HLL

- Department HR:

HOSTNAME – HR-SW, HR PRINTER

PASSWORD – HR

- Department MK:

HOSTNAME –MK-SW, MK PRINTER

PASSWORD – MK

- Department FIN:

HOSTNAME –FIN-SW, FIN PRINTER

PASSWORD – FIN

- Department BR-GWA:

HOSTNAME –BR-GWA-SW, BR-GWA PRINTER

PASSWORD – BR-GWAA

- HQ-MULTILAYER-SW:

HOSTNAME – HQ-MULTILAYER-SW

PASSWORD – HQ-ML

- HQ-MULTILAYER-SW2:

HOSTNAME – HQ-MULTILAYER-SW2

PASSWORD – HQ-MLL

- BR-MULTILAYER-SW:

HOSTNAME – BR-MULTILAYER-SW

PASSWORD – BR-ML

- BR-MULTILAYER-SW2:

HOSTNAME – BR-MULTILAYER-SW2

PASSWORD – BR-MLL

- HQ-ROUTER:

HOSTNAME – HQ-ROUTER

PASSWORD – HQ-ROUTERR

- BR-ROUTER:

HOSTNAME – BR-ROUTER

PASSWORD – BR-ROUTERR

Technologies Implemented:

- Routing Protocols:

- OSPF (Single-Area 10):

OSPF 10 is done on the routers and the multilayer switches.

IN HQ-MULTILAYER SWITCH1:

- IP routing

- OSPF 10

- Network 192.168.100.0 0.0.0.63 area 0

- Network 192.168.100.64 0.0.0.63 area 0

- Network 192.168.100.128 0.0.0.63 area 0

- Network 192.168.100.192 0.0.0.63 area 0

- Network 192.168.101.0 0.0.0.63   area 0

- Network 192.168.101.64 0.0.0.63 area 0

- Network 192.168.102.80 0.0.0.3 area 0

IN HQ-MULTILAYER SWITCH2:

- IP routing

- OSPF 10

- Network 192.168.100.0 0.0.0.63 area 0

- Network 192.168.100.64 0.0.0.63 area 0

- Network 192.168.100.128 0.0.0.63 area 0

- Network 192.168.100.192 0.0.0.63 area 0

- Network 192.168.101.0 0.0.0.63   area 0

- Network 192.168.101.64 0.0.0.63 area 0

- Network 192.168.102.84 0.0.0.3 area 0

IN BR-MULTILAYER SWITCH1:

- IP routing

- OSPF 10

- Network 192.168.101.128 0.0.0.31 area 0

- Network 192.168.101.160 0.0.0.31 area 0

- Network 192.168.101.192 0.0.0.31 area 0

- Network 192.168.101.224 0.0.0.31 area 0

- Network 192.168.102.0 0.0.0.31 area 0

- Network 192.168.102.32 0.0.0.31 area 0

- Network 192.168.102.92 0.0.0.31 area 0

IN BR-MULTILAYER SWITCH2:

- IP routing

- OSPF 10

- Network 192.168.101.128 0.0.0.31 area 0

- Network 192.168.101.160 0.0.0.31 area 0

- Network 192.168.101.192 0.0.0.31 area 0

- Network 192.168.101.224 0.0.0.31 area 0

- Network 192.168.102.0 0.0.0.31 area 0

- Network 192.168.102.32 0.0.0.31 area 0

- Network 192.168.102.96 0.0.0.31 area 0

IN HQ- ROUTER:

- OSPF 10

- Network 192.168.102.80 0.0.0.3 area 0

- Network 192.168.102.84 0.0.0.3 area 0

- Network 192.168.102.64 0.0.0.15 area 0

- Network 192.168.102.88 0.0.0.3 area 0

- Network 195.136.17.4 0.0.0.3 area 0

- Network 195.136.17.0 0.0.0.3 area 0

IN BR- ROUTER:

- OSPF 10

- Network 192.168.102.96 0.0.0.3 area 0

- Network 192.168.102.88 0.0.0.3 area 0

- Network 195.136.17.8 0.0.0.3 area 0

- Network 195.136.17.12 0.0.0.3 area 0

IN ISP1- ROUTER:

- OSPF 10

- Network 195.136.17.8 0.0.0.3 area 0

- Network 195.136.17.0 0.0.0.3 area 0

IN ISP2- ROUTER:

- Network 195.136.17.4 0.0.0.3 area 0

- Network 195.136.17.12 0.0.0.3 area 0

DEFAULT STATIC ROUTE:

- IN HQ-MULTILAYER SWITCH1:

- Ip route 0.0.0.0 0.0.0.0 192.168.102.82

- IN HQ-MULTILAYER SWITCH2:

- Ip route 0.0.0.0 0.0.0.0 192.168.102.86

- IN BR-MULTILAYER SWITCH1:

- Ip route 0.0.0.0 0.0.0.0 192.168.102.94

- IN BR-MULTILAYER SWITCH2:

- Ip route 0.0.0.0 0.0.0.0 192.168.102.98

- IN HQ- ROUTER:

- Ip route 0.0.0.0 0.0.0.0 195.136.17.2

- Ip route 0.0.0.0 0.0.0.0 195.136.17.6 70

- IN BR- ROUTER:

- Ip route 0.0.0.0 0.0.0.0 195.136.17.14

- Ip route 0.0.0.0 0.0.0.0 195.136.17.10 70

NAT/PAT :

  - NAT for internet access

  - I configured NAT on HR-ROUTER and BR-ROUTER

  - Configured port address translator on both HQ-ROUTER AND BR-ROUTER

  - So that it can be translate into public Ip address

IN HQ- ROUTER:

- Int se0/1/0 - Ip Nat outside

- Int se0/2/1 – Ip Nat outside

- Int range gig0/0-2 – Ip Nat inside

- Ip Nat inside source list 1 interface se0/2/1 overload

- Ip Nat inside source list 1 interface se0/1/0 overload

IN BR- ROUTER:

- Int se0/1/1 - Ip Nat outside

- Int se0/2/0 - Ip Nat outside

- Int range gig0/0-1 – Ip Nat inside

- Ip Nat inside source list 1 interface se0/1/1 overload

- Ip Nat inside source list 1 interface se0/2/0 overload

Access Control list:

IN HQ- ROUTER:

- Access-list 1 permit 192.168.100.0 0.0.0.63

- Access-list 1 permit 192.168.100.64 0.0.0.63

- Access-list 1 permit 192.168.100.128 0.0.0.63

- Access-list 1 permit 192.168.100.192 0.0.0.63

- Access-list 1 permit 192.168.101.0 0.0.0.63

- Access-list 1 permit 192.168.101.64 0.0.0.63

IN BR- ROUTER:

- Access-list 1 permit 192.168.101.128 0.0.0.31

- Access-list 1 permit 192.168.101.160 0.0.0.31

- Access-list 1 permit 192.168.101.192 0.0.0.31

- Access-list 1 permit 192.168.101.224 0.0.0.31

- Access-list 1 permit 192.168.102.0 0.0.0.31

- Access-list 1 permit 192.168.102.32 0.0.0.31

Site-to-Site IPSec VPN:

- IPSec VPN tunnels form HQ-ROUTER to BR-ROUTER for secure communication

VLANs & Inter-VLAN Routing:

- Inter-VLAN routing using Layer 3 switches

SSH and Port Security:

- Port security is done in server-side-site switch

DHCP, DNS, and Server Configurations:

- DHCP for dynamic IP allocation

- DNS for internal name resolution

Wireless Networking:

- Access Points for each department

- SSID configurations per VL